## Project Explanation

Our project is an IoT-based industrial fault monitoring system for a Wireless and Mobile Networks course. The main idea is to detect abnormal conditions at an industrial site and send the information over the network in real time using MQTT. The system is designed as a small IoT architecture with a sensor node, an edge gateway, a broker, and remote subscribers.

At the current stage, the Arduino side acts as the sensor node. In our prototype, fault detection is currently simulated with an ultrasonic sensor threshold: when the measured distance becomes smaller than a predefined threshold, the system marks it as a fault. A local LED is turned on when the fault flag is true and turned off when the fault flag is false. The Arduino sends JSON messages over serial only when the fault status changes, so it does not constantly spam data.

The Raspberry Pi side acts as an IoT gateway. It reads serial data from the Arduino, enriches the data with metadata such as timestamp, gateway ID, device ID, and location, logs the events locally to a CSV file, and publishes structured messages to an MQTT broker. This makes the Raspberry Pi more than just a forwarder; it behaves like an edge device that processes and distributes industrial monitoring data.

The project uses MQTT publish/subscribe communication, which is the main networking component of the system. Different MQTT topics are used for different purposes:

- events: published only when the sensor status changes, such as OK -> FAULT or FAULT -> OK

- live: published periodically every 5 seconds using the latest known sensor state

- heartbeat: periodic gateway-alive message

- status: gateway online/offline status

This topic-based design makes the project more aligned with Wireless and Mobile Networks because it demonstrates real-time messaging, topic hierarchy, QoS-based publish/subscribe communication, gateway monitoring, and remote wireless supervision.

## Current Architecture

You can describe the system as:

- Arduino Sensor Node

  - Reads sensor values

  - Detects whether the monitored value indicates a fault

  - Controls a local LED alarm

  - Sends JSON over serial only when the state changes

- Raspberry Pi Gateway

  - Receives JSON from Arduino over USB serial

  - Adds metadata and timestamp

  - Stores logs locally in fault_log.csv

  - Publishes messages to MQTT topics

  - Sends heartbeat and gateway status messages

- MQTT Broker

  - Handles publish/subscribe communication

  - Enables remote monitoring clients to receive updates

- Remote Subscriber / Dashboard

  - Can subscribe to project topics

  - Can observe event, live, heartbeat, and status messages

## Important Honesty Note

For the documents, it is better to say this clearly:

The current prototype demonstrates the communication architecture and real-time fault reporting using a threshold-based sensor input. Although the project idea targets industrial hazards such as LPG leakage and fire, the present implementation uses an ultrasonic-based fault simulation as a proof of concept. The architecture is designed so that gas and flame sensors can later replace or complement the current sensor node.

This is important because otherwise the report may overclaim compared to the current implementation.

## What Has Been Implemented

For the progress report, your friend can write that these are completed:

- Arduino fault detection logic implemented

- LED alert logic implemented

- Serial JSON communication implemented

- Interrupt-based measurement handling on Arduino implemented

- Raspberry Pi serial receiver implemented

- MQTT publishing implemented

- Structured MQTT topics implemented

- Heartbeat and gateway status messages implemented

- Periodic live-state publishing implemented

- Local CSV logging implemented

- Secure broker connection tested with HiveMQ

## Expected Next Steps

These are good future-work items:

- replace the current simulated fault input with real LPG / flame sensors

- add a dashboard for remote monitoring

- add mobile notification, email, or Telegram alerts

- support multiple sensor nodes

- measure message latency and reliability for networking evaluation

- improve fault classification such as warning / critical levels

## Software Design Summary

For the software design document, your friend can describe the software as having these modules:

- Sensor acquisition module on Arduino

- Fault evaluation module on Arduino

- Serial communication module between Arduino and Raspberry Pi

- Gateway processing module on Raspberry Pi

- MQTT communication module on Raspberry Pi

- Logging module on Raspberry Pi

- Monitoring/subscriber module on the client side

## Short Summary Paragraph

If your friend needs one short paragraph:

This project implements an IoT-based industrial fault monitoring prototype using an Arduino sensor node and a Raspberry Pi gateway. The Arduino detects abnormal conditions and sends state-change messages as JSON over serial communication. The Raspberry Pi enriches, logs, and publishes the data via MQTT using structured topics for events, live state, heartbeat, and gateway status. The system demonstrates real-time publish/subscribe communication and remote fault monitoring, making it suitable for a Wireless and Mobile Networks project.